Roubust Design Experiment Report

-Step Count Accuracy Using Smart Watch-

20206651 김준서

20215060 채현기

Contents

1. Introduction

2. Experimental Factors

2.1 Control Factors and Levels

2.2 Noise Factors

3. Experimental Design

3.1 P-Diagram

3.2 Orthogonal Array

3.3 Experimental Matrix

4. Experimental Procedure

4.1 Track Walking Setup

4.2 Step Counting Methid

4.3 Data Collection Method

5. Results and Analysis

5.1 Raw Data

5.2 Step Count Error Calculation

5.3 S/N Ratio Analysis

5.4 Main Effects Plot and Interpretation

5.5 Optimal Conditions

6. Conclusion

1. Introduction

Smartwatches have become widely used for daily activity monitoring, health tracking, and fitness management. Among the various sensing functions they provide, step counting is one of the most fundamental metrics, serving as the basis for estimating calorie consumption, walking distance, and overall physical activity levels. Despite their popularity, smartwatch step-count accuracy can vary significantly depending on how the device is worn and the physical conditions under which walking occurs. Factors such as arm movement, wearing side, shoe type, and even the specific smartwatch model may influence the sensor signals used to detect steps.

Because users wear smartwatches in diverse daily environments, it is essential to identify the conditions under which step-count measurements remain accurate and robust against external variations. This study applies a Taguchi robust design approach to evaluate how different wearing conditions affect step-count accuracy during walking. By systematically examining four controllable factors—arm motion (fixed or natural swing), wearing side (left or right arm), footwear type (slippers or sneakers), and smartwatch model (Galaxy Watch or Apple Watch)—the experiment aims to determine which combination provides the most reliable step-count performance.

To measure real-world walking behavior in a controlled setting, each trial consists of walking one full lap around a standard athletic track. The step count recorded by the smartwatch is compared with the actual number of steps taken to quantify measurement error. The experiment introduces walking speed as a noise factor to test how sensitive each condition is to environmental variation. Through this analysis, the study seeks to identify a wear configuration that minimizes error while maintaining stable performance under different walking conditions.

2. Experimental Factors

This study investigates four controllable factors that may influence the accuracy of smartwatch step-count measurements during walking. Each factor is tested at two levels to allow for a Taguchi L8 orthogonal array design. In addition, walking speed is introduced as a noise factor to evaluate the robustness of each condition under varying walking environments.

2.1 Control Factors and Levels

| Factor | Description | Level 1 | Level 2 |
| --- | --- | --- | --- |
| A – Arm Motion | Degree of arm movement during walking | Fixed (arm held steady) | Natural swing |
| B – Wearing Arm | Arm on which the smartwatch is worn | Left arm | Right arm |
| C – Footwear Type | Type of shoes worn while walking | Slippers | Sneakers |
| D – Watch Model | Smartwatch device used for measurement | Galaxy Watch | Apple Watch |

These four factors are selected because they represent common real-world variations in how users wear and interact with their smartwatches. Understanding their effects helps determine a configuration that yields more consistent and accurate step-count data.

2.2 Noise Factor

| Noise Factor | Level 1 | Level 2 |
| --- | --- | --- |
| Walking Speed | Slow walking | Fast walking |

Walking speed naturally varies among users and across situations. Introducing this as a noise factor allows the experiment to evaluate how sensitive each condition is to external variability. The aim is to identify a factor combination that maintains low measurement error even when walking conditions change.

3. Experimental Designs

The purpose of the experimental design is to systematically evaluate how the selected control factors and noise factor influence the accuracy of smartwatch step-count measurements. A Taguchi robust design framework is adopted to efficiently analyze multiple factors while minimizing the total number of experiments.

3.1 P-Diagram

The P-diagram is used to conceptually describe the system, including the inputs, control factors, noise factors, and the resulting output (step-count error).

Input: Actual steps taken during one lap of the track

Control Factors: Arm motion, wearing arm, footwear type, watch model

Noise Factor: Walking speed (slow vs. fast)

Mechanism: Smartwatch accelerometer and motion-detection algorithms

Output: Step-count error (difference between measured and actual steps)

This diagram helps identify how each factor contributes to variations in the output and guides the choice of experimental conditions.

3.2 Orthogonal Array Selection

Since four control factors are examined at two levels each, a Taguchi L8 (2⁴) orthogonal array is selected.

This design reduces the number of required experiments from 16 (full factorial) to 8, while still allowing reliable estimation of the main effects.

Each row of the L8 array corresponds to one experimental condition (a combination of factor levels).